William J. Hughes Technical Center

Advanced Electronic Flight Strip (AEFS)

Suitability Test Plan

| Document # | TFDM-B3-TPL-001 |
| --- | --- |
| Version # | 1.0 |
| Version Date | August 29, 2016 |

Federal Aviation Administration (FAA)

William J. Hughes Technical Center (WJHTC)

Air Traffic Systems (ATS) Test & Evaluation Services Division ANG-E5

Atlantic City International Airport

New Jersey 08405

Approval Signatures

Document Title:	AEFS  Suitability Test Plan

DOCUMENT VERSION CONTROL

| Version | Description of Change | Date |
| --- | --- | --- |
| 0.1 | Initial Draft Document | 07/13/2016 |
| 0.2 | Incorporated comments | 07/18/2016 |
| 0.3 | Incorporated comments | 08/01/2016 |
| 0.4 | Incorporated comments | 08/03/2016 |
| 0.5 | Incorporated comments | 08/08/2016 |
| 1.0 | Final release for signatures | 08/29/2016 |

Table of Contents

1	Introduction	6

1.1	Background	6

1.2	Purpose	6

1.3	Scope	6

2	Reference Documents	7

2.1	FAA Documents	7

2.2	Other Government Documents	7

3	AEFS System Description	8

3.1	AEFS System Overview	8

3.2	Interfaces Overview	9

4	Test Program Description	11

4.1	Test and Evaluation Approach	11

4.2	Test Coordination and Conduct	11

4.3	Suitability Test Script Descriptions	12

4.3.1	Test Facilitator  Script	14

4.3.2	Clearance Delivery (CD) Script	14

4.3.3	Ground Control (GC) Script	14

4.3.4	Local Control South West flow (LC SW) Script	14

4.3.5	Local Control Class B Airspace (LC CBA) Script	14

4.3.6	Traffic Manager (TM 32)	14

4.3.7	Traffic Manager Flight Data (TM FD)	14

4.3.8	Traffic Manager Cab Coordinator (TM CC) Script	14

4.3.1	Arrival Metering (AM) Script	15

4.3.2	Server Cabinet  Script	15

4.3.3	Test Setup  Script	15

4.4	Data Collection and Analysis Methods	15

4.5	WJHTC AEFS Test Environment	15

4.6	Test and Analysis Tools	17

5	Test Management	18

5.1	Test Management Organization	18

5.1.1	Test Program Roles And Responsibilities	18

5.1.2	Integrated Test Team (ITT)	18

5.2	Personnel Resource Requirements	18

5.3	Training	18

5.3.1	Test Participant Training	18

5.4	Configuration Management (CM)	18

5.4.1	AEFS CM	18

5.4.2	Test Bed Configuration Management	19

5.5	Suitability Entrance Criteria	19

5.6	Suitability Test Execution	19

5.7	Discrepancy Reports (DRs)	19

5.8	Suitability Test Exit Criteria	20

5.9	Suitability Assessment Caucus	20

5.10	Suitability Test Report	20

5.11	Suitability Test Schedule	20

5.12	Planning, Considerations, and Limitations	20

6	Acronyms	21

7	Appendix A: AEFS Suitability Observation/Feedback Form	23

8	Appendix B: AEFS Suitability Questionnaire	24

9	Appendix C: AEFS Suitability High Traffic Test Scripts and M Demo	25

# Introduction

## Background

The Advanced Electronic Flight Strip (AEFS) is a Federal Aviation Administration (FAA)-developed; FAA owned and operated system being deployed as an early implementation solution for the Terminal Flight Data Manager (TFDM) system to improve Air Traffic Control Tower (ATCT) controller performance and operational effectiveness as part of the FAA’s NextGen Implementation Strategic Plan (NISP). The AEFS system is currently deployed as a prototype system at two (2) airports Phoenix (PHX) ATCT and Cleveland (CLE) ATCT and Newark (EWR) , San Francisco (SFO), Las Vegas (LAS), and Charlotte (CLT) are currently undergoing implementation.

The ANG-E59 test team has been tasked by the TFDM AJM-224 Program Management Office (PMO) with developing a Test Plan for evaluating the suitability of the AEFS system at for field operations. To achieve this, the ANG-E59 test team will conduct Test and Evaluation (T&E) activities at the William J. Hughes Technical Center (WJHTC) AEFS laboratory. T&E activities will include the development of test cases to capture the suitability issues identified by Air Traffic Control (ATC) personnel at the sites, develop test procedures to be used for Key Site testing, and to dry run the test procedures to ensure they are complete and ready for ATC personnel to execute.

## Purpose

The purpose of the AEFS Suitability Test Plan is to describe the various planning and preparation activities required to perform Suitability testing at the Site and at the WJHTC AEFS Lab. This plan defines the testing to be accomplished, the test script approaches, test procedure execution, discrepancy management, and the strategy for making a suitability call on the AEFS National Software baseline.

## Scope

The AEFS Program Requirements Document (PRD), dated December 18, 2014, defines the requirements under test for this activity. Key Site Suitability testing will utilize system users such as ATC personnel, Technical Operations (Tech Ops) personnel, and other NAS end users. Their involvement will be used to assess the suitability and effectiveness of the AEFS system within the National Airspace System (NAS) environment.  In addition, any fixes to Discrepancy Reports (DRs) deemed critical and/or system stability issues identified during previous testing will be included.

# Reference Documents

## FAA Documents

Table 2-1 FAA Program Documents

| Document Title | Version |
| --- | --- |
| AEFS Program Requirements Document (PRD) PRD-PMO-02-TFDM-14-001 | Rev- December 18, 2014 |
| AEFS System Prototype National Baseline Test Plan and Procedures | V 2.0, October 30, 2014 |
| AEFS Regression Test Plan for Cleveland TFDM-C3-PLN-001 | V1.0, February 10, 2015 |
| AEFS Operators Manual | V2.03, May 14, 2015 |
| AEFS Technical Instruction Book, TI 6131.16 | V0.1, 09/15/2015 |

## Other Government Documents

Table 2-2 Other Government Documents

| Document Title | Version |
| --- | --- |

# AEFS System Description

## AEFS System Overview

The AEFS system is a NAS program being deployed as part of the TFDM Early Implementation (EI) program at select FAA terminal locations. AEFS provides controllers with a real time, secure, efficient and effective means of distributing flight strips electronically to support air traffic control functions in the air traffic control towers and eliminates the need for paper flight strips.

The concept of the AEFS system is to electronically replace the paper Flight Strip (FS) and its many uses, improve data entry capabilities, and enhance situational awareness.  The AEFS system receives arrival/departure flight plan data from the Flight Data Input/Output (FDIO) Personal Computer Replacement Control Unit (PC-RCU) interface and provides an interface for ATCT personnel that will allow them to manually enter and amend the flight strip data.  Flight data will be displayed via Touch Screen Displays (TSDs) and will provide all of the information that would normally print on a flight strip.  Each TSD is configurable for one of eight (8) positions based on operational need: Clearance Delivery (CD), Ground Metering (GM), Arrival Metering (AM), Ground Control (GC), Local Control (LC), Traffic Management (TM), Quality Assurance (QA), and ATC Operations (ATCOPS).

AEFS will receive flight plan data from the FDIO PC-RCU and distribute the flight strips to the applicable operational positions.  Flight data is displayed in the form of Electronic Flight Strips (EFS) organized in configurable queues.  Air Traffic Control (ATC) personnel will be able to view, amend, update, transfer, progress, and discard flight strips by using the TSD as well as view and modify strips on another operational position.  Amendments to flight data will be passed to En Route Automation Modernization (ERAM) via the FDIO interface.  Changes to the flight strips will appear highlighted to indicate new information or information required to be issued to the pilot before departure.

AEFS will replace the functionality of paper Flight Strips (FSs) in ATCT operations. AEFS will provide the benefit of always displaying up-to-date flight plan data on the EFS, including automatically updated revisions.

Figure 3-1 illustrates the operational concept and information flow for AEFS that is typical at a field site.

Figure 3-1: AEFS Concept and Information Flow

## Interfaces Overview

AEFS processes and distributes Electronic Flight Strip Transfer System (EFSTS) amendments. AEFS has the capability to send and receive information from the Flight Data Input/Output system (FDIO). AEFS also interfaces with EFSTS and Airport Resource Management Tool (ARMT) to support the sharing of flight strip information electronically.

AEFS system connects to single or dual Flight Data Input/Output (FDIO) Personal Computer-Remote Control Unit (PC-RCU) racks. AEFS system is adapted in ERAM as a FDIO peripheral device: Remote Alpha Numeric Keyboard (RANK), Cathode-Ray Tube (CRT), and Remote Flight Strip Printer (RFSP). Through these connections AEFS is able to communicate to the NAS ERAM, which provides controllers with standard operating flight strip-related commands. In the event of an AEFS outage facilities have the ability to print paper strips.

AEFS is also connected to the EFSTS. This connection allows strips to be printed at the site-adapted departure controller positions at the Terminal Radar Approach Control (TRACON). AEFS transmits typical EFSTS pre-scan data to the EFSTS system, which includes: runway, heading and related data. Similar to the AEFS-FDIO connection, AEFS virtually emulates the EFSTS Local Control (LC) and Ground Control (GC) scanner devices.

At facilities with ARMT, the AEFS interface connection is primarily for AEFS to receive arrival flight plan data hand-off coordination from the TRACON. The ARMT system accepts data from AEFS system input, which defines the hand-off parameters within ARMT. ARMT uses radial distance and altitude parameters to establish a boundary around the arrival airport facility supported by AEFS. As arrival flights penetrate the ARMT boundary parameter, AEFS is sent a hand-off message indicating to which arrival threshold/runway the flight plan is sequenced.

Figure 3-2 illustrates the AEFS system interfaces in a single tower topology.

Figure 3-2: AEFS System Interfaces

# Test Program Description

## Test and Evaluation Approach

The primary objective of the suitability testing is to determine whether the AEFS system is suitable, if the AEFS system meets the user needs, and if it can be used efficiently with minimal disruption to ATC operations. The suitability test event is designed to stress the AEFS system in order to check for system performance and lag times before deploying the AEFS as a baseline system. The ANG-E59 test team will develop the test scripts as a “High Traffic Day in the Life” Site perspective. Suitability testing will be performed by selected ATC personnel and assisted by AJV Subject Matter Experts (SMEs) and the ANG-E59 test team. ATC personnel will operate the AEFS to move traffic at a very high rate while assisting personnel, i.e. AJV SMEs and other program stakeholders, will preempt the scripts by calling out certain ATC actions.

Suitability testing will occur in two phases.  The first phase will be conducted at the WJHTC AEFS Lab with ATC and Tech Ops personnel working together and will include induced failures and degraded modes operations. The Maintenance (M) Demo from the AEFS Development Test (DT) Procedure Book 4 will be used by the Tech Ops personnel while ATC personnel will be exercising the AEFS Suitability High Traffic scripts. A Cleveland Center (ZOB) _Suitability scenario was developed to be run on the ERAM In The Box (EIB) for this phase and includes up to two hundred twenty-five (225) arriving and departing flights within a six (6) hour timeframe. There will be approximately forty-five (45) FSs in the system every 15 minutes for the test participants to manipulate.

In the event that the first phase of testing is successful, a second phase of testing will occur at the Site ATCT for ATC personnel only. This test phase will use test flight plans that will be input into the operational NAS from the Direct User Access Terminal System (DUATS) and will be displayed for manipulation and modification by controllers exercising the AEFS Suitability High Traffic scripts, these test scripts can be used at any ATCT facility. Flight plans from live operations will be used for arrival flight operations to ensure the ARMT system interoperates with the AEFS system correctly.

## Test Coordination and Conduct

Following the successful completion of software installation at the Site ATCT and prior to any test activities, the ANG-E59 TFDM Test Director will conduct an In-Briefing to the test participants to ensure the system is stable and ready for operations. There will be daily in-briefs, (if needed), describing the test events and the coordination of resources and participants.  The TFDM Test Director will conduct daily Out-Briefs, (if needed), to review test conduct logs and any resulting Discrepancy Reports (DRs). These Out-Briefs will ensure all test events were captured and any carryover items that will impact upcoming test activities are identified.  These Out-Briefs will help determine whether the AEFS is suitable for use in the NAS.

During the Suitability Test conduct, AEFS Observation/Feedback Forms, refer to Appendix A, will be provided to the test participants to capture any potential issues discovered during the test event. Any DRs generated during the test event will be processed by the AJM-224 PMO Change Control Board (CCB) process. An AEFS Suitability Questionnaire, refer to Appendix B, will also be provided to the test participants immediately following test completion. The questionnaires will be used to capture subjective data and are based on tasks typical at their facility. This subjective data will be used to augment the objective data collected from the test artifacts. Participants are asked to take accurate notes on any abnormal issues that they incur which would affect their answers to the questionnaire.

Following the completion of the Suitability Testing, a Suitability Assessment Caucus will be conducted by the ANG-E59 TFDM Test Director to review the test results so a determination of whether the AEFS system is suitable for use in the NAS can be made by the test participants, AJV personnel, and other stakeholders. In addition, any DRs captured will be analyzed for system defects and the responses from the Suitability questionnaire(s) will be addressed by the test participants to address operational impacts.

The principal questions that the test participants will need to consider during the Suitability Assessment caucus are:

- Is the AEFS system suitable for inclusion within the NAS Operational Environment?

- Can the AEFS system be used by ATCT personnel with minimal interruptions to ATC operations?

The answers to these questions along with the supporting data will be used by the ATCT test participants to make the Suitability decision at the Suitability Assessment Caucus.

## Suitability Test Script Descriptions

The AEFS Lab at the WJHTC will be divided into two test strings; String 4 will be configured with CLE adaptation will incorporate eight (8) touchscreen display positions (CD, FD, GC, LC SW, CBA, TM32, CC, AM) that will be the primary user test string.  The CLE String will also be configured to run with the EIB, FDIO and the PC-RCU which will emulate real flight data exchange.  String 3 will be configured with PHX adaptation and will incorporate eight (8) touchscreen display positions (CD, GM, GC, LC1, LC2, TM1, TM2, and TM3) that will be used to calculate Mean Time Before Failure (MTBF). The PHX String will use the FDIO simulator for flight data.

String 4 will consist of 8 AEFS positions; each position user will be provided a script of repetitive duties, e.g. moving FS, amending flight plans, searching for flights.  Test Facilitators, utilizing scripts, will intermittently direct individual/group AEFS positions to perform specific AEFS functions (external stressors).

Except for the setup scripts all scripts are run in parallel or concurrent by an individual tester and test facilitator.  Testing will be repeated. The CD, FD, GC, LC SW, CBA, TM32, CC, AM, and the server cabinet scripts are repeated until testing is completed, while the test facilitator script(s) may preempt or interrupt the scripts sequence to perform an action at the request of the Test Facilitator.  There may be more than one Tester for each Test Facilitator. Each Test Facilitator will have a copy of their script. The sequence of the script will continue once the action of the test facilitator’s script is completed. A Tester will perform additional steps as requested by the Test Facilitator.  A Test Logger will capture the time, stressor #, position, and preempted step in a test timeline log.  The diagram pictured below gives a high level visualization of the process.

Figure 4-2: Suitability Test Execution

### Test Facilitator  Script

The Test Facilitator script contains additional actions that the tester will perform as requested by the Test Facilitator. The additional actions that the Test Facilitator will select from are listed in the External Stressor Table. A Test Logger will capture the time, stressor #, position, and preempted step in a Test Timeline log.

### Clearance Delivery (CD) Script

The script for the CD position will expand flight strips, changes routes, and acknowledge the magenta highlights. The CD will be asked to manually assign an FRC, acknowledge the FRC, and assign Gates, ATIS codes, and IALT via quick assignment buttons. Further exercises will include modifying altitudes and aircraft types, adding remarks to the FSs, and conducting searches on FSs.

### Ground Control (GC) Script

The GC position will be asked to drag FSs into queues and re-sequence them within the queues. The GC position will also be asked to acknowledge TMIs and enter release times. The GC will also be asked to assign/amend Gates, ATIS codes, and IALTs using the quick assignment buttons.  The GC will also create, select, and transfer FS to other positions.

### Local Control South West flow (LC SW) Script

The LC SW position will be asked to assign Clear to Land and Hold Shorts to Arrival Strips.  The LC will also assign LUAW, CFTO, and DPT to Departure Strips.. The LC will be asked to drag FSs to various queues.

### Local Control Class B Airspace (LC CBA) Script

The LC CBA position will be asked to pair queues to other positions and re-pair/change queue types i.e. Search or Control. The LC CBA position will also be asked to create VFR FSs and transfer FSs to various positions. The LC CBA position will also discard multiple FSs.

### Traffic Manager (TM 32)

The TM 32 position will be asked to amend the TMIs and also re-sequence FSs in the arrival queue.

### Traffic Manager Flight Data (TM FD)

The TM FD position will be asked to transfer multiple FSs to various positions, to expand FSs, change routes, gates, ATIS, IALTs and acknowledge the magenta highlights.  The TM FD position will also be asked to close and reopen the position as well as change TMI fixes.

### Traffic Manager Cab Coordinator (TM CC) Script

The TM CC position will be asked to add and delete queues as well as reconfigure the position. The TM CC will also re-sequence FSs in the arrival queue.

### Arrival Metering (AM) Script

The AM Position will be asked to add and delete queues. The AM will also change the sorting of a queue, create arrival FSs and transfer to local’s arrival queue.

### Server Cabinet  Script

The Server Cabinet script contains the actions to restart the software on the primary and secondary servers and to ensure that all Thin Clients reflect the server status.

### Test Setup  Script

The Test Setup script must be run first in order to set the TM TM32, TM FD, TM CC, LC SW, CBA, GC SW, CD, AM positions in the High Traffic EWR Tower configuration. Each position will arrange the queues on their AEFS GUI according to the script.

## Data Collection and Analysis Methods

Controller feedback and system issues will be documented and processed using ANG-E59’s existing DR process.  Artifacts for the event will consist of running test logs to document any issues or anomalies, the collection of executed test procedures, collecting Suitability Observation/Feedback forms, and test status reports summarizing each day’s test events. A final AEFS Test Report will be delivered separately that will document all the test results and the Key Site Suitability resolution.

## WJHTC AEFS Test Environment

In preparation for the Suitability testing that will be conducted at a ATCT Site, the ANG-E59 test team will dry run the Suitability Test Procedures in the AEFS test lab at the WJHTC.  The AEFS test infrastructure consists of real and simulated system components to provide an end to end test environment that replicates robust field operations as close as possible.

The AEFS test lab consists of port servers, switches, thin client PCs, and Touch Screen Displays (TSDs). The AEFS servers receive arrival/departure flight plans from the Flight Data Input/Output (FDIO) Personal Computer Replacement Control Unit (PC-RCU) through the port server. The flight plan data is stored in an AEFS database and is converted into electronic flight strips which are displayed to the users on the TSD.

For sites that have the Departure Spacing Program (DSP) or the Airport Resource Management Tool (ARMT), the AEFS lab can be configured to include or exclude these systems. However for dry running the Suitability test procedures the AEFS lab will be configured to interface with the ERAM In-A-Box (EIB) to enable AEFS to receive and send Flight Data (FD) through FDIO and the PC-RCU. There will be three new AEFS EIB workstations to interface and configure the EIB environment.  The EIB will interface with the Simulation Driver RADAR Recorder (SDRR) to exchange flight data with AEFS. The ERAM Data Distribution System (EDDS) receives flight plans from EIB through the SDRR and forwards it to the DSP system.  AEFS will send DSP Scan Messages to the DSP Towers. STARS will exchange Flight and Radar Data with SDRR and ARMT. ARMT will in turn send airport surface data to AEFS.  The AEFS Test Bed consists of port servers, switches, thin client PCs, and Touch Screen Displays (TSDs). The AEFS servers receive arrival/departure flight plans from the FDIO Personal Computer Replacement Control Unit (PC-RCU) through the port server. The flight plan data is stored in an AEFS database and is converted into electronic flight strips which are displayed to the users on the Touch Screen Displays.

The first phase of Suitability Testing will be conducted at the WJHTC AEFS Lab.  ATC and Tech Ops personnel will be working together as they normally would at a field facility. This phase will include induced failures and degraded modes operations to ensure that the AEFS service availability does not adversely impact AT operations. The Maintenance (M) Demo from the AEFS DT Procedure Book 4 will be used by the Tech Ops personnel while ATC will be exercising the AEFS Suitability High Traffic scripts. A ZOB_Suitability scenario was developed for this phase and includes up to two hundred twenty-five (225) arriving and departing flights within a 2 hour timeframe.

The test configuration at the WJHTC is depicted in Figure 4-3 and consists of the systems listed in Table 4-3.

Figure 4-3: AEFS Test Environment

Table 4-3:  AEFS Lab Interfacing Systems

| # | System Name | Description | Intended Use |
| --- | --- | --- | --- |
| 1 | ERAM in a BOX (EIB) | EIB allows users to simulate the ERAM operational environment that are not dependent on a real infrastructure. EIB provides the capabilities needed to test the operational application software and supports functional verification of interfaces between software applications, adaptation data, interfacility communications, and System Analysis Recording (SAR) data collection without the use of any real operational test beds. | EIB will be used to provide a two-way communication with FDIO via SDRR for flight data exchange. |
| 2 | Simulation Driver and RADAR Recorder (SDRR) | SDRR is a simulation injection tool which allows injection of flight plan data. | SDRR will be used to inject flight plans for processing by AEFS and DSP. |
| 3 | Flight Data Input Output (FDIO) | FDIO provides a conduit to receive and transmit flight plan data from En Route Automation Modernization (ERAM). | FDIO will be used to exchange flight data for between EIB and AEFS |
| 4 | ERAM Data Distribution System (EDDS) | EDDS is a database that contains Flight Plan data from all 21 Air Route Traffic Control Centers (ARTCC’s) | EDDS will be used to supply the EWR flight plans for the development of the AEFS-DSP scenario. |
| 5 | Departure Sequencing Program (DSP) | DSP is a coordination and planning tool that uses pertinent air traffic information from airports equipped with the system; along with other information from filed flight plans, to coordinate departures by spacing aircraft more evenly over a fix. | The DSP system will be used to verify requirements for the AEFS scanner event messages |
| 7 | En-Route Data Distribution System (EDDS) Simulator | EDDS is a data distribution system that performs a set of services for ERAM and client applications. It provides ERAM with a single point of presence to facilitate the exchange of Common Message Set (CMS) messages between ERAM and other clients. It also has an Application Program Interface (API) that allows clients to access EDDS services. | To provide ERAM interface with FDIO |
| 8 | Standard Terminal Automation Replacement System (STARS) | STARS is a Terminal Radar Approach Control (TRACON) facility that is used to manage air traffic immediately around major airports | STARS will be used to pass flight data to AEFS |
| 9 | Electronic Flight Strip Transfer System (EFSTS) | The EFSTS will exchange data in accordance with NAS-IC-82518256-01. | EFSTS will be used |
| 10 | Airport Resource Management Tool (ARMT) | ARMT provides the Traffic Manager with tools to monitor and analyze arrival and departure demand, based on flight plan and event data received from the STARS interfaces and from EFSTS flight event data received from towers via the EFSTS server. | ARMT will be used to pass airport surface data to AEFS |

## Test and Analysis Tools

Microsoft Excel

# Test Management

## Test Management Organization

The AEFS system is managed by the TFDM PMO, AJM-224.  For the Developmental Test (DT) phase of the test program, the Air Traffic Management Branch, ANG-E59, was tasked with the responsibility for the development of test procedures, verification testing, and status reporting of the AEFS system to the TFDM PMO.  Additionally, during DT, the FAA WJHTC Air Traffic Management Branch is the lead Government test organization that oversees the DT planning, conduct, and reporting.  Following DT, the TFDM test team is responsible for the planning and conduct of AEFS Suitability testing and reporting test results directly to the TFDM PMO.

### Test Program Roles And Responsibilities

The Air Traffic Management Branch Manager, who administratively reports to the Air Traffic Systems Test & Evaluation Services Division Manager, oversees the efforts of the TFDM Test Director.  The TFDM Test Director is responsible for leading and directing the activities of the TFDM test team.  The TFDM test team is comprised of Government and contractor test personnel, operational field test support personnel, and other Government test participants from program stakeholder organizations. In addition to managing the DT and Suitability test team, the TFDM Test Director also oversees the DT test activities conducted by the TFDM test team.

### Integrated Test Team (ITT)

The Automation Engineering Branch, AJM-244, and the Air Traffic Management Branch, ANG-E59, are supporting this test effort.

## Personnel Resource Requirements

Suitability testing will be performed by selected ATC personnel and assisted by AJV Subject Matter Experts (SMEs).

## Training

### Test Participant Training

The expectation is that the test team participants will be recruited based on their knowledge of the AEFS system, and will require no training on the use of the system.  A test plan familiarization briefing will be conducted to familiarize participants with test procedures, schedule and expectations.  Configuration Management (CM)

### AEFS CM

The Automation Engineering Branch, AJM-244, will maintain configuration-controlled hardware and software CM.

### Test Bed Configuration Management

A configuration audit will take place prior to formal testing.  The system will be installed with a National Software (SW) Baseline build that has been delivered by the Automation Engineering Branch, AJM-244

## Suitability Entrance Criteria

- Equipment and software is installed and available

- Build under test is available and under CM

- Selected Site personnel are available

- Build fixes are available and were evaluated in the lab with no critical discrepancy documented

- Suitability Test Plan has been reviewed and approved

- Suitability Test Procedures have been reviewed and dry runs have been completed

## Suitability Test Execution

A Suitability Test In-Brief will be presented to the test  Site personnel prior to the start of Suitability testing. The In-Brief is conducted to discuss the logistics and the test strategy of the Suitability Test event. Once the In-Brief has been conducted, the execution of the test procedures will commence. Each test participant will execute the test procedures according to the specific instructions for his/her role in the test. The test team will be available for assistance and will maintain a test conduct log of all issues, anomalies, and deviations during testing. A Test Participant and/or Test facilitator should identify themselves to the Test Loggers once an issue is discovered.  If an issue occurs which stops the execution of the test, the Test Director, Test Facilitators, and Test Participants will assess the impact and determine when the test can resume.

## Discrepancy Reports (DRs)

Any anomalies found such as a hardware failure or software problem during Suitability testing will be documented as a DR and discussed by the CCB.   All DRs will be prioritized in accordance to the following priority definitions:

- Priority 1 is assigned to a problem that prevents the accomplishment of an operational, mission-critical or mission-essential capability, or jeopardizes safety, security or other requirements designated as CPRs.

- Priority 2 is assigned to a problem that adversely affects the accomplishment of an operational, mission-critical or mission-essential capability and no work-around solution is known.

- Priority 3 is assigned to a problem that adversely affects the accomplishment of an operational, mission-critical or mission-essential capability, but a work-around solution is known.

- Priority 4 is assigned to a problem that results in a user/operator inconvenience or annoyance, but does not affect a required operational, mission-critical or mission-essential capability.

- Priority 5 is assigned to any other problem/defect not described above (e.g., system documentation errors.

## Suitability Test Exit Criteria

- All planned Suitability test scripts have been executed to completion

- All Suitability test results including test conduct logs and DRs have been documented

- All test artifacts have been collected and assembled for discussion for the Suitability Outbrief, i.e. DRs, As Run Test Scripts, Suitability Observation/Feedback forms, AEFS Suitability Questionnaires, and test conduct logs.

## Suitability Assessment Caucus

A Suitability Assessment Caucus will be held at the conclusion of the Key Site Suitability test with the test participants in attendance to present and review all information and results from the Suitability testing. The Caucus chaired by the ANG-E59 TFDM Test Director, is conducted to determine the Suitability of the AEFS by having the ATCT test participants provide their responses to the Suitability questions. Each Suitability issue will be addressed, all test results will be reviewed and DRs will be analyzed for operational impacts. All new issues generated during Suitability testing will be addressed by the test participants to assess the impact to the Suitability questions. To support the resolution a review of the results from the test participant’s questionnaire will be included.

## Suitability Test Report

The Suitability Test Report will be delivered to the TFDM PMO thirty (30) days after test completion and will contain the results from the Suitability Assessment Caucus.

## Suitability Test Schedule

High level test schedule for the test preparation and  site testing will be developed and distributed to participants prior to the test.

## Planning, Considerations, and Limitations

Need to coordinate with facilities with the  Site for the Suitability Testing participants.

# Acronyms

| ACRONYM | DESCRIPTION |
| --- | --- |
| AEFS | Advanced Electronic Flight Strips |
| AM | Arrival Metering |
| ARMT | Airport Resource Management Tool |
| ARTCC | Air Route Traffic Control Center |
| ATC | Air Traffic Control |
| ATCOPS | Air Traffic Control Operations |
| ATCT | Air Traffic Control Tower |
| CCB | Configuration Control Board |
| CD | Clearance Delivery |
| CM | Configuration Management |
| DR | Discrepancy Report |
| DSP | Departure Sequencing Program |
| DT | Development Testing |
| EFD | Electronic Flight Data |
| EFS | Electronic Flight Strip |
| EFSTS | Electronic Flight Strip Transfer System |
| ERAM | En Route Automation Modernization |
| EUI | Early User Involvement |
| EWR | Newark International Airport |
| FAA | Federal Aviation Administration |
| FDIO | Flight Data Input / Output |
| FID | Final Investment Decision |
| FS | Flight Strips |
| GC | Ground Control |
| GM | Ground Metering |
| IOC | Initial Operating Capability |
| ITT | Integrated Test Team |
| LC | Local Control |
| NAS | National Airspace System |
| OT | Operational Test |
| PRD | Program Requirements Document |
| QA | Quality Assurance |
| STARS | Standard Terminal Automation Replacement System |
| TGF | Target Generator Facility |
| TM | Traffic Management |

| ACRONYM | DESCRIPTION |
| --- | --- |
| TRACON | Terminal Radar Approach Control |
| TSD | Touch Screen Display |
| T&E | Test and Evaluation |
| TFDM | Terminal Flight Data Manager |
| TRR | Test Readiness Review |
| VRTM | Verification Requirement Traceability Matrix |
| WJHTC | William J. Hughes Technical Center |

# Appendix A: AEFS Suitability Observation/Feedback Form

| Position: | [ ] CD; [ ] GH; [ ] AM; |  | [ ] LC; [ ] LA; [ ] TM; |  | [ ] ATCOPS; [ ] QA; [ ] Other _____________; |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name: (optional) |  |  |  |  |  |  |
| Phone/E-mail: (optional) |  |  |  |  |  |  |
| - Date of Observation: |  |  |  |  |  |  |
| - System Time: |  |  |  |  |  |  |
| - Description of Observation/Feedback: Example format: (a) Statement of Issue (b) Detailed steps to recreate; (c) Expected Result; (d) Actual Result;; (e) Workaround (if applicable) i.e. how you got out of situation to be able to continue |  |  |  |  |  |  |
| - Affected Position(s) (check all that apply): | [ ] CD; [ ] GM; [ ] AM; |  | [ ] LC; [ ] LA; [ ] TM; |  | [ ] ATCOPS; [ ] QA; [ ] Other ____________; |  |
| - Affected Hardware (check all that apply): | [ ] Touchscreen/Monitor; [ ] Workstation/Thin Client; [ ] KVM; [ ] Switch; |  |  | [ ] Database Application Server (DBS); [ ] Data Storage Server (DSS); [ ] FTI; [ ] Other _______________; |  |  |
| - Affected Applications (check all that apply): | [ ] N/A; | [ ] Client; |  | [ ] DBS; |  | [ ] DSS; |
|  | [ ] Other _______________; |  |  |  |  |  |
| - Affected Interfaces (check all that apply): | [ ] N/A; | [ ] FDIO; |  | [ ] EFSTS; |  | [ ] ARMT; |
|  | [ ] Other _______________; |  |  |  |  |  |
| - Reproducible? | [ ] N/A; | [ ] Yes; |  | [ ] No; |  | [ ] Intermittent |
| - Priority (Observer’s Recommendation): | [ ] 1-Critical; [ ] 2-Major/High; [ ] 3-Medium; [ ] 4-Handled Procedurally; [ ] 5-Annoyance/Low; |  |  |  |  |  |

# Appendix B: AEFS Suitability Questionnaire

NAME (optional): ____________________________

POSITION (optional):  _______________________

Instructions: Thinking about the testing you just completed, please indicate to what extent you agree with each statement using the rating scale below:

1= Strongly Disagree   2=Disagree   3=Agree    4=Strongly Agree

| 1. | I was able to use the AEFS TSDs to create, amend, and transfer FSs without interrupting ATC operations. | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| 2. | I was able to assign strip markings to FSs | 1 | 2 | 3 | 4 |
| 3. | I was able to use the AEFS virtual and physical keyboards for amending FS data. | 1 | 2 | 3 | 4 |
| 4. | I was able to use the AEFS queues and bays for organizing, searching, moving, and offsetting FSs. | 1 | 2 | 3 | 4 |
| 5. | I was able to create and delete queues | 1 | 2 | 3 | 4 |
| 6. | I was able to create a quick assignment bar specific to my position | 1 | 2 | 3 | 4 |
| 7. | I was able to modify my positions settings |  |  |  |  |
| 8 | I was able to determine status for specific aircraft. | 1 | 2 | 3 | 4 |
| 9. | I was able to view and understand the data on the FS | 1 | 2 | 3 | 4 |
| 10. | I was able to transfer the FS to other positions with the correct data in a timely manner | 1 | 2 | 3 | 4 |
| 11. | I was able to reconfigure the positions easily | 1 | 2 | 3 | 4 |
| 12. | I was able to reconfigure the runway and airspace easily | 1 | 2 | 3 | 4 |
| 13. | I was able to use the All button for different actions, i.e. move, transfer, discard, drag and drop. | 1 | 2 | 3 | 4 |
| 14. | I was able to use the quick action buttons for various operations, i.e. H/S, Revert, LUAW, Vehicle, VEH CLS, and Disable. | 1 | 2 | 3 | 4 |
| 15. | I was able to utilize the buttons on the FS status bar | 1 | 2 | 3 | 4 |
| 16. | I was able to create a memo and send to other positions | 1 | 2 | 3 | 4 |
| 17. | I was able to adjust the M&C buttons |  |  |  |  |
| 18. | The AEFS was stable for operations | 1 | 2 | 3 | 4 |
| 19. | The AEFS was accurate and timely during operations |  |  |  |  |
| 20. | The AEFS was easy to recover | 1 | 2 | 3 | 4 |

- For any item rated ‘2’ or below, please provide an explanation at the conclusion of the questionnaire using your notes to substantiate the rating.

# Appendix C: AEFS Suitability High Traffic, Stress, Lag, Performance Test Scripts and M Demo

| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
| --- | --- | --- | --- | --- | --- | --- |
| Test Description: There is 1 AEFS Test Facilitator script (callout ATC actions), 8 AEFS operational test scripts, 1 test setup script, and 1 server cabinet script. The test setup script must be run first. All other scripts thereafter are run in parallel or concurrent by an individual Tester and Test Facilitator. The CD, GC, LC SW, CBA, TM32, FD, CC, AM, and the server cabinet scripts are repeated over and over, while the Test Facilitator script(s) may preempt or interrupt the scripts sequence to perform an action at the request of the Test Facilitator. There may be more than one Test Facilitator. Each Test Facilitator will have a copy of their own script. The sequence of the script will continue once the action of the Test Facilitator’s script is completed. All Test Loggers will document the results when a script is preempted. The diagram pictured to the right gives a high level visualization of the process. |  |  |  |  |  |  |
| Pre Test Setup: The TM TM32, TM FD, TM CC, LC SW, CBA, GC SW, CD, AM position configurations must be created using the ATCOPS Position prior to executing this test. More than 1 ATCOPS Positions may be used to expedite creation of positions. Follow steps 4 to 30 to create each position configuration. |  |  |  |  |  |  |
|  | On Server 1 and Server 2, set the AM Position to ON in the AEFS Server Administration screen. | AM Position is set to ON in the AEFS Server Administration screen |  |  |  |  |
|  | Verify the AEFS System has DBServices running on Server 1 & 2 with no errors displayed. | DBServices 1 & 2 are running without any errors displayed. |  |  |  |  |
|  | Verify Server 1 is the primary server for DBServices. | Server 1 is the primary server. |  |  |  |  |
| EWR AEFS Position Setups (TM TM32, TM FD, TM CC, LC SW, CBA, GC SW, CD, AM (if available)) |  |  |  |  |  |  |
| TM32 TM-based Position. Line up paired to LC; Taxi Paired to GC; Auto Track STOP 5 fixes; Auto Track APREQ 5 fixes; Default. Auto Track DSP 5 fixes; GI. :FFQ is a Search |
|  | Verify there is an AEFS TM32 TM Position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | TM Position online and configured correctly to EWR specs. |  |  |  |  |
|  | At the TM32 TM Position, verify there are 5 APREQ, DSP, and STOP fixes in each queue. If not, then create them using KORD, KBOS, KLAX, KCLT, KATL, KSFO, KDFW, or KPAL. | 5 Fixes created for APREQ, DSP and STOP without errors. |  |  |  |  |
|  | AT the TM32 TM Position, verify the Lineup queue is paired with LC Line up. If not, then pair it. | TM Line up paired with LC Line up. |  |  |  |  |
|  | AT the TM32 TM Position, verify the Taxi queue is paired with GC Taxi. If not, then pair it. | TM Taxi paired with GC Taxi. |  |  |  |  |
| FD TM-based Position. Line up paired to LC; Taxi Paired to GC; UAL Paired to GC; Other Paired to GC; Default. ASQ paired to GC. :FFQ is Control |
|  | Verify there is an AEFS TM FD Position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | FD Position online and configured correctly. |  |  |  |  |
|  | AT the TM FD Position, verify the Lineup queue is paired with LC Line up. If not, then pair it. | FD Line up paired with LC Line up. |  |  |  |  |
|  | AT the TM FD Position, verify the Taxi queue is paired with GC Taxi. If not, then pair it. | FD Taxi paired with GC Taxi. |  |  |  |  |
|  | AT the TM FD Position, verify the UAL queue is paired with GC UAL. If not, then pair it. | FD UAL paired with GC UAL. |  |  |  |  |
|  | AT the TM FD Position, verify the Other queue is paired with GC Other. If not, then pair it. | FD Other paired with GC Other. |  |  |  |  |
|  | AT the TM FD Position, verify the ASQ queue is paired with GC ASQ. If not, then pair it. | FD ASQ paired with GC ASQ. |  |  |  |  |
| CC TM-based Position. Default; Line up paired to LC; Taxi paired with GC. Board paired to LC. : FFQ is Control |
|  | Verify there is an AEFS TM CC Position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | CC in online and configured correctly. |  |  |  |  |
|  | AT the TM CC Position, verify the Lineup queue is paired with LC Line up. If not, then pair it. | CC line up paired with LC line up. |  |  |  |  |
|  | AT the TM CC Position, verify the Board queue is paired with LC Board. If not, then pair it. | CC board paired with LC board. |  |  |  |  |
|  | AT the TM CC Position, verify the Taxi queue is paired with GC Taxi. If not, then pair it. | CC taxi paired with GC taxi. |  |  |  |  |
| LC SW LC-based Position. Arrivals; Board; Line Up. : FFQ is Control |
|  | Verify there is an AEFS LC SW Position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | LS SW online and configured correctly. |  |  |  |  |
|  | At the LC SW Position, verify the Arrivals queue has a DMZ on for 5 FSs. If not, reconfigure the queue to have the DMZ on with 5 FSs (Lock). |  |  |  |  |  |
|  | At the LC SW Position, verify the Lineup queue has a DMZ on for 5 FSs. If not, reconfigure the queue to have the DMZ on with 5 FSs (Lock). |  |  |  |  |  |
| CBA LC-based Position. Default; Board paired to LC (User will change paired queue to search queue during this test); Taxi paired to GC (User will change paired queue to control queue during this test). : FFQ is Control |
|  | Verify there is an AEFS CBA LC Position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | CBA LC online and configured correctly. |  |  |  |  |
|  | AT the CBA Position, verify the Board queue is paired with LC Board. If not, then pair it. | CBA board paired with LC board. |  |  |  |  |
|  | AT the CBA Position, verify the Taxi queue is paired with GC Taxi. If not, then pair it. | CBA taxi paired with GC taxi. |  |  |  |  |
| GC SW GC-based Position. Line up paired to LC; Taxi; UAL; Other; Arrivals. ASQ. : FFQ is Control |
|  | Verify there is an AEFS GC SW position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | GC SW online and is configured correctly. |  |  |  |  |
| - V | At the GC SW Position, verify line up queue is paired with LC Line up. If not, then pair it. | GC SW line up paired with LC line up. |  |  |  |  |
| CD CD-based Position. Taxi paired to GC; UAL; Other; ASQ; Proposed. Search. : FFQ is Search |
|  | Verify there is an AEFS CD position online and is configured as in the table above. If it is not online, then create, configure and restart the position. | CD online and is configured correctly. |  |  |  |  |
|  | At the CD Position, verify the Taxi queue is paired with GC. If not, then pair it. | CD taxi paired with GC taxi. |  |  |  |  |
| AM AM-based Position. Arrivals (default); LC Arrivals paired to LC Arrivals; GC Arrivals paired with GC Arrivals.. :FFQ is a Search |
|  | Verify there is an AEFS AM position online (if available) and is configured as in the table above. If it is not online, then create, configure and restart the position. Skip step if not available. | AM online and is configured correctly. |  |  |  |  |
|  | At the AM Position, verify LC Arrivals is paired with LC Arrivals. If not, then pair it. | AM LC Arrivals paired with LC Arrivals. |  |  |  |  |
|  | At the AM Position, verify GC Arrivals is paired with GC Arrivals. If not, then pair it. | AM GC Arrivals paired with GC Arrivals. |  |  |  |  |
| Test Setup and Initialization for EWR |  |  |  |  |  |  |
|  | If testing is being performed at the WJHTC AEFS lab, then Eram-in-a-box (EIB) needs to be started by the Test Team 17 minutes prior to continuing; see Erik Lorenzo or Matthew Sanderlin. If testing is being performed in the field, then skip this step. | EIB started without errors. |  |  |  |  |
|  | At the TM32 Position, verify using the M&C window that following positions are on: CD, GC, LC, AM. | CD, GC, LC and AM buttons are green indicating on. All other positions are off. |  |  |  |  |
|  | At the position where arrivals are defaulting too, transfer 50 arrival FS to the LC SW arrivals queue. | CD transferred 50 FS to LC SW without delay or errors. |  |  |  |  |
|  | At the CD Position transfer 10 departure FS to LC SW. | CD transferred 10 FS to LC SW without delay or errors. |  |  |  |  |
|  | At the CD Position, transfer 30 FSs to the GC SW Position. | CD transferred 30 FS to GS CW without delay or errors. |  |  |  |  |
|  | Verify all test engineers have their test case scripts ready. | Test scripts available to all test engineers. |  |  |  |  |
|  | Establish how long the test case will be run concurrently at all positions. | Test case duration established and recorded. |  |  |  |  |
|  | Start concurrent position actions at all manned positions. | Test case begins. Time is recorded. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance TEST Facilitator Script. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: A Test Facilitator may request an additional step at any time from the External Stressor Scenario X table listed below. A Tester will perform additional steps as requested by the Test Facilitator. A Test Logger will capture the time, stressor #, position, and preempted step in a Test Timeline log. Test Facilitator steps do not have to be performed in order. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP 1A: Using the Table “External Stressor Scenario X” listed below, select a stressor and a position to perform the stressor. | Scenario stressor selected from table below. |  |  |  |  |
|  | Verify the Test Logger records in the Test Timeline Log file the following information: - Time - Stressor # (Callout) - Position - Preempted step - Pass or fail of stressor | A Test Logger accurately reflects the actions of the stressor on the position and recorded the information in a Test Timeline Log. Example Test Timeline log: Test Facilitator #1 Log. Time; Stressor; Position; Preempted Step; Pass/Fail. 08:11:11; 20; CD; 6; Pass. 08:25:09; 11; TM; 5; Pass. 08:45:12; 12; LC; 7; Pass. |  |  |  |  |
|  | STEP 2B: Repeat Steps at STEP 1A to STEP 2B until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance CLEARANCE DELIVERY (CD) Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The CD position will expand flight strips, changes routes, and acknowledge the magenta highlights. The CD will be asked to manually assign an FRC, acknowledge the FRC, assign Gates, ATIS codes, and IALT via quick assignment buttons. Further exercises will include modifying altitudes and aircraft types, adding remarks to the FS, and conducting searches on FSs. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP A: Expand a FS. | FS expanded. |  |  |  |  |
|  | Click on route and change the route using the Swap Routes button on FS (must be domestic flight). | Route changed on FS without delay or errors. |  |  |  |  |
|  | Acknowledge the magenta lines on the route of the FS and acknowledge the FRC check box. | Magenta Line and FRC acknowledged without delay or errors. |  |  |  |  |
|  | Assign Gate, ATIS code, and IALT using the quick assignment buttons. | Gate, ATIS code, and IALT assigned without delay or errors. |  |  |  |  |
|  | Transfer the FS to GC using the GC NLA button on FS. | FS transferred to GC without delay or errors. |  |  |  |  |
|  | Select/mark a departing FS. | FS selected without delays. |  |  |  |  |
|  | If “DCL/CPDLC” Wilco quick action button is available then press it, else select any random quick action button. | Quick action assigned to FS without delays or errors. |  |  |  |  |
|  | Wait 5 seconds before continuing to the next step. |  |  |  |  |  |
|  | STEP B: Repeat Steps at STEP A to STEP B 10 times. | Steps repeated without delays or errors. |  |  |  |  |
|  | Transfer up to 40 FS to the FD Position. | CD transferred 40 FSs to FD without delay or errors. |  |  |  |  |
|  | Perform a random search for any FSs. | Search performed without delay or errors. |  |  |  |  |
|  | Select and move 30 or less FSs from queue to queue. | FSs move from queue to queue without errors or delays. |  |  |  |  |
|  | Expand any departing FS. | FS expanded without delays or errors. |  |  |  |  |
|  | Select the FDIO button, and using the FDIO keyboard, make a route amendment on the expanded FS. | Route of FS changed without delays or errors. |  |  |  |  |
|  | Click on the small “FRC” yellow box on FS to acknowledge it. | “FRC” acknowledged without delays or errors. |  |  |  |  |
|  | Modify requested ALT on FS. | ALT modified without delays or errors. |  |  |  |  |
|  | Modify requested Type on FS. | Type modified without delays or errors. |  |  |  |  |
|  | Enter “Medavac” to remarks on FS. Then collapse the expanded FS. | “Medavac” entered into remarks field without delays or errors. |  |  |  |  |
|  | STEP B2: Repeat Steps at STEP A to STEP B2 until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance GROUND CONTROL (GC) SW Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The GC position will be asked to drag FSs into queues and resequence them within the queues. The GC position will also be asked to acknowledge TMIs and enter release times. The GC will also be asked to assign/amend Gates, ATIS codes, and IALTs using the quick assignment buttons. The GC will also create, select, and transfer FS to other positions. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP C: Drag FS to Taxi queue. | FS dragged to Taxi without delay or errors. |  |  |  |  |
|  | Assign Runway, Gate, ATIS code, and IALT using the quick assignment buttons. | Gate, ATIS code, and IALT assigned without delay or errors. |  |  |  |  |
|  | Acknowledge any APREQ, or DSP TMIs. Then enter current time in the info field. | TMIs acknowledged on FS without delay or errors. Current time entered in info box. |  |  |  |  |
|  | Acknowledge any APREQ, DSP, or STOP TMIs. | TMIs acknowledged on FS without delay or errors. |  |  |  |  |
|  | Drag FS to Line up queue. | FS dragged to line up without delay or errors. |  |  |  |  |
|  | Wait 2 seconds before continuing to the next step. |  |  |  |  |  |
|  | Rearrange FSs in Taxi, and Lineup paired queues. | FSs rearranged without delays or errors. |  |  |  |  |
|  | STEP D: Repeat Steps at STEP C to STEP D 10 times. | Steps repeated without delays or errors. |  |  |  |  |
|  | Expand any departure FS. | FS expanded without delays. |  |  |  |  |
|  | Amend the ATIS code, Gate, and IALT on FS. Then collapse the expanded FS. | FS amendments accepted without delays or errors. |  |  |  |  |
|  | Create Vehicle Strip | Vehicle strip created without delays or errors. |  |  |  |  |
|  | Select up to 5 FSs | FSs selected without delays or errors. |  |  |  |  |
|  | Transfer all FSs to the CD for re-routes via any one of the three methods available. | FSs transferred to CD without errors or delays. |  |  |  |  |
|  | STEP D2: Repeat Steps at STEP C to STEP D2 until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance LOCAL CONTROL (LC) SW Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The LC SW position will be asked to assign Clear to Land and Hold Shorts to Arrival Strips. The LC will also assign LUAW, CFTO, and DPT to Departure Strips.. The LC will be asked to drag FSs to various queues, |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP E: Drag Departure FS from the Departures queue to Board queue. Select FS and press the LUAW NLA. | FS dragged to Board without delay or errors. FS in LUAW status without delay or errors. |  |  |  |  |
|  | Select an arrival FS. Click the NLA button to clear FS to land. | Arrival FS selected. Arrival FS Cleared to land |  |  |  |  |
|  | On the departure FS, change the FREQ. | FREQ accepted without errors or delays. |  |  |  |  |
|  | On the arrival FS, assign VAP. (if available) If not, use quick action buttons to change Runway. | VAP assigned without errors or Runway changed. |  |  |  |  |
|  | If necessary, on the departure FS, expand and acknowledge any APREQ, DSP, , or STOP TMIs. | TMIs acknowledged on FS without delay or errors. |  |  |  |  |
|  | On the departure FS, press the NLA CFTO on the FS. | FS CFTO without delay or errors. |  |  |  |  |
|  | On the arrival FS, drag FS to the Board queue. | Arrival FS dragged to board without delay or errors. |  |  |  |  |
|  | On the departure FS, click on the DPT button twice to depart the FS. | FS departed without delay or errors. |  |  |  |  |
|  | On the arrivals FS, assign H/S to FS using quick assignment buttons. | Arrival FS assigned H/S without delay or errors. |  |  |  |  |
|  | On the departure FS, delete the FS by pressing on the NLA X button. | FS discarded without delay or errors. |  |  |  |  |
|  | On the arrival FS, Transfer FS to GC using NLA button. | Arrival FS transferred to GC without delay or errors. |  |  |  |  |
|  | STEP F: Repeat Steps (slow and steady) at STEP E to STEP F until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |
| EWR AEFS High Traffic, Stress, Lag, Performance LOCAL CONTROL (LC) CBA Position. |  |  |  |  |  |  |
| Note: The LC CBA position will be asked to pair queues to other positions and re-pair/change queue types i.e. Search or Control. The LC CBA position will also be asked to create VFR FSs and transfer FSs to various positions. The LC CBA position will also discard multiple FSs. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP G: Verify the Board queue is paired with LC Board. If not, then re-pair it. | CBA Board paired with LC without delay or errors. |  |  |  |  |
|  | Verify the Taxi queue is paired with GC Taxi. If not, then re-pair it. | CBA Taxi paired with GC Taxi without delays or errors. |  |  |  |  |
|  | Wait 10 seconds before continuing to the next step. |  |  |  |  |  |
|  | Discard between 1 and 10 FSs. | FSs discarded without errors or delays to the system (LC position). |  |  |  |  |
|  | Transfer between 1 and 10 FSs to CD. | FSs transferred without delays or errors. |  |  |  |  |
|  | Change arrival sequencing by moving FSs in the arrivals queue. | FSs sequencing changed without delays or errors. |  |  |  |  |
|  | If feature available, create VFR Helicopter FS & transfer to LC. If not, create a Vehical FS and transfer it to the LC. | Helicopter or Vehical FS created and transferred to LC without delays or errors. |  |  |  |  |
|  | Change the Board queue to a search queue. | CBA un pairs Board without delay or errors. |  |  |  |  |
|  | Change the Taxi queue to a control queue. | CBA un pairs Taxi without delay or errors. |  |  |  |  |
|  | Wait 10 seconds before continuing to the next step. |  |  |  |  |  |
|  | STEP H: Repeat Steps at STEP G to STEP H until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance Traffic Management (TM) TM32 Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The TM 32 position will be asked to amend the TMIs and also re-sequence FSs in the arrival queue. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP H1: Observe all actions being performed by all other positions. | TM observed actions without delay or errors. |  |  |  |  |
|  | Change the 5 APREQ, DSP, and STOP fixes in each queue. Use KORD, KBOS, KLAX, KCLT, KATL, KSFO, KDFW, or KPAL, etc. | 5 Fixes created for APREQ, DSP and STOP without errors. |  |  |  |  |
|  | Wait 60 seconds before continuing to the next step. |  |  |  |  |  |
|  | Change arrival sequencing by moving FSs in the arrivals queue. | FSs sequencing changed without delays or errors. |  |  |  |  |
|  | STEP H2: Repeat Steps at STEP H1 to STEP H2 until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance Traffic Management (TM) FD Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The TM FD position will asked transfer multiple FSs to various positions, to expand FSs, change routes, gates, ATIS, IALTs and acknowledge the magenta highlights. The TM FD position will also be asked to close and reopen the position as well as change TMI fixes. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP J: Transfer up to 20 FS to the CBA Position. (If there are 20 available at the time, else transfer a few). | FS transferred to CBA without delay or errors. |  |  |  |  |
|  | Transfer up to 20 FS back to CD. (If there are 20 available, else transfer a few). | FS transferred without delay or errors. |  |  |  |  |
|  | Expand a FS. | FS expanded. |  |  |  |  |
|  | Click on route and change the route using the Swap Routes button. | Route changed on FS without delay or errors. |  |  |  |  |
|  | Acknowledge the magenta lines on the route of the FS. | Magenta Line acknowledged without delay or errors. |  |  |  |  |
|  | Change Gate, ATIS code, and IALT using the quick assignment buttons. | Gate, ATIS code, and IALT assigned without delay or errors. |  |  |  |  |
|  | Close the position using the minimization button. | FD position closed without delay or errors. |  |  |  |  |
|  | Open a TM FD Position. | FD position opened without delay or errors. |  |  |  |  |
|  | Wait 1 minute before continuing to the next step. |  |  |  |  |  |
|  | STEP K: Repeat Steps at STEP J to STEP K until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance Traffic Management (TM) CC Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The TM CC position will be asked to add and delete queues as well as reconfigure the position. The TM CC will also re-sequence FSs in the arrival queue. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP L: Observe all actions being performed by all other positions. | CC observed actions without delay or errors. |  |  |  |  |
|  | Randomly reconfigure the position by adding and deleting queues. | CC position reconfigured without delay or errors. |  |  |  |  |
|  | Reconfigure the position to the original configuration. | CC Position reconfigured without delay or errors. |  |  |  |  |
|  | Wait 10 seconds before continuing to the next step. |  |  |  |  |  |
|  | Change arrival sequencing by moving FSs in the arrivals queue. | FSs sequencing changed without delays or errors. |  |  |  |  |
|  | STEP M: Repeat Steps at STEP L to STEP M until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance Arrival Metering (AM) Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The AM Position will be asked to add and delete queues. The AM will also change the sorting of a queue, create arrival FSs and transfer to local’s arrival queue. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | STEP N: At the AM Position, observe all queues and the actions being performed by all other positions. | AM observed actions without delay or errors. |  |  |  |  |
|  | Create Arrival FS and drag to LC paired arrival queue. | Arrival FS created without errors or delays. |  |  |  |  |
|  | Add a queue | Queue added without errors or delays. |  |  |  |  |
|  | Change queue sorting (rearrange FS in queue) | FS sorted without delays. |  |  |  |  |
|  | Delete queue. | Queue deleted without errors. |  |  |  |  |
|  | Wait 10 seconds before continuing to next step. |  |  |  |  |  |
|  | STEP O: Repeat Steps at STEP N to STEP O until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

| EWR AEFS High Traffic, Stress, Lag, Performance Server Cabinet Position. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Note: The Server Cabinet script contains the actions to restart the software on the primary and secondary servers and to ensure that all Thin Clients reflect the server status. |  |  |  |  |  |  |
| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
|  | Verify SLE has Wireshark running on both servers 1 & 2. If not, make a log entry that is not running and continue to next step. | - Wireshark is running on both servers 1 and 2 without errors or conflicts with the AEFS program. |  |  |  |  |
|  | STEPS 1A: On Primary Server 1, wait approximately 30 minutes, then using the Start icon, select the Restart option. | - Server 2 becomes the primary server. - Server 1 restarts - Activities and actions at all positions - All positions continue to operator without delays or errors. |  |  |  |  |
|  | When Server1 restarts, Login and restart the AEFS Software selecting - Operational Mode – Operational - Primary Server - Yes - Logging Mode – Error, Client, EFSTS, FDIO Messages - FDIO Inter-Connections – Yes - EFSTS Inter-Connections – No - ARMT Inter-Connections – No | Server 1 restarts |  |  |  |  |
|  | On Server 1, set the AM Position to ON in the AEFS Server Administration screen. | AM Position is set to ON in the AEFS Server Administration screen |  |  |  |  |
|  | Ensure all Thin clients reflect Server status. From the M&C screen on the CD, GC, CBA, TM32, TM FD, TM CC, and AM thin clients verify that Server 2 is listed as the Primary Server and Server 1 is active in backup mode. | Server 2 listed as Primary Server. Server 1 is active backup mode. |  |  |  |  |
|  | From the AEFS System Options tab, verify the Servers reflect the changed status | Server 2 listed as Primary Server. Server 1 is active in backup mode. |  |  |  |  |
|  | Wait approximately 30 minutes from the recovery of Server 1 in Secondary mode and press the Windows icon on Server 2, select the Restart option. | - Server 1 becomes the primary server. - Server 2 restarts - Activities and actions at all positions continue without delays or errors. - Server 2 recovers and is now the backup server. |  |  |  |  |
|  | When Server 2 restarts, Login and restart the AEFS Software selecting - Operational Mode – Operational - Primary - No - Logging Mode – Error, Client, EFSTS, FDIO Messages - FDIO Inter-Connections – Yes - EFSTS Inter-Connections – No - ARMT Inter-Connections – No | Server 2 restarts |  |  |  |  |
|  | On Server 2, set the AM Position to ON in the AEFS Server Administration screen. | AM Position is set to ON in the AEFS Server Administration screen |  |  |  |  |
|  | Ensure all Thin clients reflect Server status. From the M&C screen on the CD, GC, CBA, TM32, TM FD, TM CC, and AM thin clients verify that Server 1 is listed as the Primary Server and Server 2 is active in backup mode. | Server 1 is listed as the Primary Server. Server 2 is active in backup mode. |  |  |  |  |
|  | From the AEFS cabinet, verify the Servers reflect the changed status | Server 1 is listed as the Primary Server. Server 2 is active in backup mode. |  |  |  |  |
|  | STEPS 1B: Repeat Steps at STEP 1A to STEP 1B until test is completed (hours, duration, set for test case). | Steps repeated without delay or errors. |  |  |  |  |

External Stressor Scenario X Table

| Item # | External Stressor Scenario X X indicates suggested stressor to be exercised at position. | TM TM32 | TM FD | TM CC | LC LC SW | LC CBA (LC Assist) | GC GC SW | CD | AM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flight Strip Transfer: have user utilize different methods of moving FS |  |  |  |  |  |  |  |  |  |
|  | NLA | X | X | X | X | X | X | X | X |
|  | Transfer button | X | X | X | X | X | X | X | X |
|  | Drag and Drop | X | X | X | X | X | X | X | X |
| Multiple button: utilize the multiple button for different actions |  |  |  |  |  |  |  |  |  |
|  | Move | X | X | X | X | X | X | X | X |
|  | Transfer | X | X | X | X | X | X | X | X |
|  | Drag and Drop | X | X | X | X | X | X | X | X |
|  | Offset | X | X | X | X | X | X | X | X |
|  | Discard | X | X | X | X | X | X | X | X |
| All button: have user utilizing the All button for different actions |  |  |  |  |  |  |  |  |  |
|  | Move | X | X | X | X | X | X | X | X |
|  | Transfer | X | X | X | X | X | X | X | X |
|  | Drag and Drop | X | X | X | X | X | X | X | X |
|  | Offset | X | X | X | X | X | X | X | X |
|  | Discard | X | X | X | X | X | X | X | X |
| Sorting: have user sort FS within bay |  |  |  |  |  |  |  |  |  |
|  | Sort FS manually | X | X | X | X | X | X | X | X |
|  | Sort FS using sorted button selections | X | X | X | X | X | X | X | X |
| New FS Button: have user create a new FS |  |  |  |  |  |  |  |  |  |
|  | New FS Button: have user create a new FS (departure) |  |  |  |  |  |  | X |  |
|  | New FS Button: have user create a new FS (arrival) |  |  |  |  | X |  |  | X |
|  | New FS Button: have user create a new FS (hangar, vehicle) |  |  |  |  |  | X | X |  |
| Memo Button: |  |  |  |  |  |  |  |  |  |
|  | Create a memo | X |  |  |  |  |  |  |  |
|  | Edit a memo | X |  |  |  |  |  |  |  |
|  | Insert memo in bay |  |  |  |  |  |  |  |  |
|  | Create and send memo to another position | X |  |  |  |  |  |  |  |
|  | Create a placard |  |  | X | X |  | X |  |  |
|  | Create a MSG |  |  |  |  |  |  |  |  |
| Strip Marking: have user assign strip marking |  |  |  |  |  |  |  |  |  |
|  | Runway |  |  |  | X |  |  |  |  |
| Item # | External Stressor Scenario X | TM TM32 | TM FD | TM CC | LC LC SW | LC CBA (LC Assist) | GC GC SW | CD | AM |
|  | FRC |  |  |  |  |  |  | X |  |
|  | quick assign keys |  |  |  | X | X | X | X | X |
|  | hold short |  |  |  | X |  | X |  |  |
|  | Intersection departure |  |  |  | X |  | X |  |  |
|  | block 10-18 times |  |  |  |  |  |  |  |  |
| Quick Action Bar & Buttons: have user utilize quick action buttons |  |  |  |  |  |  |  |  |  |
|  | H/S |  |  |  | X |  | X |  |  |
|  | Revert |  |  |  | X |  |  |  |  |
|  | LUAW |  |  |  | X |  |  |  |  |
|  | Vehicle |  |  |  |  |  | X |  |  |
|  | VEH CLS |  |  |  |  |  |  |  |  |
|  | DISABLE |  |  |  | X |  |  |  |  |
|  | ABORT |  |  |  | X |  |  |  |  |
|  | CROSS |  |  |  | X |  |  |  |  |
| Flight Strip Status Bar: have user utilize the different status bar buttons |  |  |  |  |  |  |  |  |  |
|  | Priority | X |  | X | X |  | X | X |  |
|  | EMERG |  |  |  | X |  | X |  |  |
|  | YC Flight Strip Marking |  |  |  |  |  |  |  |  |
|  | V Flight Strip Marking |  |  |  |  |  |  |  |  |
|  | MA Flight Strip Marking |  |  |  | X |  |  |  |  |
|  | NoCLR Flight Strip Marking | X |  | X |  |  |  | X |  |
|  | New Flight Strip Window |  |  |  |  |  |  |  |  |
|  | NoDP Flight Strip Marking |  |  |  |  |  |  |  |  |
|  | DPOK Flight Strip Marking |  |  |  |  |  |  |  |  |
|  | DSP Flight Strip Marking | X |  | X |  |  | X |  |  |
|  | APEQ Flight Strip Marking | X |  | X |  |  | X |  |  |
|  | STOP Flight Strip Marking | X |  | X |  |  | X |  |  |
|  | FR Send Full Route to Messages Window |  |  |  |  |  |  | X |  |
|  | PEN Flight Strip Marking | X | X | X |  |  | X | X |  |
| Create / Delete Queues: have user create and delete queues: |  |  |  |  |  |  |  |  |  |
|  | Control Queue Creation | X | X | X |  | X |  |  |  |
|  | Default Queue | X | X | X |  | X |  |  |  |
|  | Search Queue Creation | X | X | X |  | X |  |  |  |
|  | Paired Queue Creation | X | X | X |  | X |  |  |  |
|  | Paired Queue Modification | X | X | X |  | X |  |  |  |
|  | RESTR Queue Creation |  |  |  |  |  |  |  |  |
| Item # | External Stressor Scenario X | TM TM32 | TM FD | TM CC | LC LC SW | LC CBA (LC Assist) | GC GC SW | CD | AM |
|  | TM PARR Queue Arrival Strips |  |  |  |  |  |  |  |  |
|  | TM Position GI Queue | X |  |  |  |  |  |  |  |
|  | Drag Assign Queue |  |  |  |  |  |  |  |  |
|  | Create RWY Queue |  |  |  |  |  |  |  |  |
|  | Create Sorted Queue |  |  |  |  |  |  |  |  |
|  | Create DMZ within Queue |  |  |  | X |  |  |  |  |
| M&C Buttons: Have the user adjust the M&C buttons: |  |  |  |  |  |  |  |  |  |
|  | PDC redirect: TM only | X | X | X |  |  |  |  |  |
|  | Active Positions button: TM only | X | X | X |  |  |  |  |  |
|  | Closure Distribution button: TM only | X | X | X |  |  |  |  |  |
|  | Drag/Drop Assignment Re-sequence button: |  |  |  |  |  |  |  |  |
|  | Use Quick Assignment buttons: |  |  |  |  |  |  |  |  |
|  | Change Quick Assignment buttons: |  |  |  |  |  |  |  |  |
|  | Paired Queue Footer button: |  |  |  |  |  |  |  |  |
|  | Show Clock: change the show clock option to reveal the move button |  |  |  |  |  |  |  |  |
|  | Auto Flight Strip Removal buttons: TM only | X | X | X |  |  |  |  |  |
|  | Departed Flight Strip Removal buttons: TM only | X | X | X |  |  |  |  |  |
|  | Drag/Drop Highlight Retention: Single FS and Multiple FS |  |  |  |  |  |  |  |  |
|  | FS Status Tag Indicators buttons |  |  |  |  |  |  |  |  |
|  | RECAT - disable recat for test: TM only | X | X | X |  |  |  |  |  |
|  | TMI Notifications buttons: TM only | X | X | X |  |  |  |  |  |
|  | FS Time Out Settings buttons: TM only | X | X | X |  |  |  |  |  |
|  | Expanded FS Behavior: |  |  |  |  |  |  |  |  |
|  | Color Palette button |  |  |  |  |  |  |  |  |
|  | Handoff Range and Altitude: TM only | X | X | X |  |  |  |  |  |
|  | Mouse wheel Scroll Speed |  |  |  |  |  |  |  |  |
| Other Buttons: Have the user use buttons: |  |  |  |  |  |  |  |  |  |
|  | Move button: after clock is removed from display have user utilize the move button to move FS between queues |  |  |  |  |  |  |  |  |
|  | Auto Locate: use auto locate to find a FS |  |  |  |  |  |  |  |  |
|  | FDIO Button: have user make amendments using the FDIO keyboard |  |  |  |  |  |  | X |  |
|  | Discard button: have user discard FS | X | X | X |  | X | X | X |  |
|  | Departed button: have user depart FS |  |  |  | X |  |  |  |  |
| Item # | External Stressor Scenario X | TM TM32 | TM FD | TM CC | LC LC SW | LC CBA (LC Assist) | GC GC SW | CD | AM |
|  | History button: have user review history for each entry |  |  |  | X |  |  |  |  |
|  | Messaging buttons: have user select and clear GI, Msg's, Error | X |  |  |  |  |  | X |  |
|  | RWY CFG button (TM only): open and close runways utilizing the House Closure and City Closure buttons | X | X | X |  |  |  |  |  |
|  | FFQ button: have user utilize the FFQ- drag and drop or search. | X | X |  |  |  |  | X |  |
|  | FS Orientation Toolbar buttons: have user adjust the orientation of FS utilizing the different options of this toolbar. |  |  |  |  |  | X | X |  |
|  | Scratch Pad: have user write data in the scratch pad, add sheets, discard sheets. | X |  | X |  |  |  | X |  |
|  | Dynamic Assignment button: have user change quick assignment buttons | X |  |  | X |  | X | X |  |
|  | Show Configuration button: have user select a pre-configured quick assignment layout |  |  |  |  |  | X | X |  |
|  | Rwy Configuration button: to open and close runways | X |  |  |  |  |  |  |  |
|  | Reconfiguration Toolbar | X |  | X |  |  |  |  |  |
|  | Create Queue: create a queue | X | X | X |  | X |  |  |  |
|  | Delete Queue: delete a queue | X |  | X |  | X |  |  |  |
|  | FFQ: have user change from active queue with FS to search queue without removing FS | X |  | X |  | X |  |  |  |
|  | Position CFG - select a pre-configured configuration |  | X |  | X |  | X |  |  |
|  | Reset: set the queue configuration back to default for that position |  | X |  | X |  | X |  |  |
|  | Quick Action |  |  |  | X |  | X |  |  |
|  | Aborted Flight Strip |  |  |  | X |  |  |  |  |
|  | Flight Strip, Quick Assignment Bars, Status and Quick Action Bars |  |  |  |  |  |  |  |  |
|  | DMZ Enabled Owner Queue |  |  |  | X |  | X |  |  |
|  | DMZ Enabled Paired Position |  |  |  | X |  | X |  |  |
|  | TM Remove Fix tracking for ESP,SWAP or STOP | X |  |  |  |  |  |  |  |
|  | EXIT Program and Restart: have user exit the program and restart | X | X | X | X | X | X | X |  |
|  | Position Recovery: have user recover the position | X | X | X | X | X | X | X |  |
|  | Minimize Application button | X | X | X | X | X | X | X | X |
|  | Synchronize Screen button | X | X | X | X | X | X | X | X |
|  | Brightness Control | X | X | X | X | X | X | X | X |
|  | Change TM options in DBS server TM Options Tab | X | X | X | X | X | X | X | X |
|  | Flight Strip History Abort Button | X | X | X | X | X | X | X | X |
|  | Reconfiguration Toolbar | X | X | X | X | X | X | X | X |
| Item # | External Stressor Scenario X | TM TM32 | TM FD | TM CC | LC LC SW | LC CBA (LC Assist) | GC GC SW | CD | AM |
|  | Runway Selection- from GUI | X | X | X | X | X | X | X | X |
|  | Runway Selection | X | X | X | X | X | X | X | X |
|  | Runway Selection- Partial from Intersection | X | X | X | X | X | X | X | X |
| Positions: Have the user modify positions settings: |  |  |  |  |  |  |  |  |  |
|  | Modifying Active Positions to GC | X | X | X | X | X | X | X | X |
|  | Modifying Position CFG for Midnight Configuration to GC | X | X | X | X | X | X | X | X |
|  | Modifying Active Positions to LC | X | X | X | X | X | X | X | X |
|  | Modifying Position CFG for Midnight Configuration to LC | X | X | X | X | X | X | X | X |
|  | Modifying Active Positions to GC | X | X | X | X | X | X | X | X |
|  | Modifying Position CFG for Midnight Configuration to GC | X | X | X | X | X | X | X | X |
|  | Modifying Active Positions to CD | X | X | X | X | X | X | X | X |
|  | Modifying Position CFG for Midnight Configuration to CD | X | X | X | X | X | X | X | X |

| Test Timeline Log |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Facilitator # |  |  |  |  |
| Time | Stressor # | AEFS Position | Preempted Step, Notes | Pass/Fail |

Maintenance (M)-Demo

Note:  Selected test cases from the procedure below will be used for the Tech Ops Suitability testing @ the WJHTC

| Step | Procedure/Operation | Expected Results | Observed Results | Pass | Fail | DR# |
| --- | --- | --- | --- | --- | --- | --- |
|  | Refer to Appendix F – AEFS System Network Topology for use with this section. |  |  |  |  |  |
|  | A technician who is familiar with the AEFS is available and on standby. A fault will be injected to the LRUs without the knowledge of the technician. The technician then will be called in and asked to resolve the problem. The technician will have to identify and replace the correct filed LRU. The time to identify the problem, replace the LRU and get the system up and running will be measure. Time for Logistic tasks such as getting a new LRU will be discarded |  |  |  |  |  |
|  | Turn off the primary Data Base Server and measure the time to replace the LRU_________________ During Fail-Over move several FS between positions. Note: Be sure to properly identify FS for tracking purposes | The AEFS is still operating |  |  |  |  |
|  | Disconnect a LAN cable from a switch and measure the time to replace the LRU________________ During Fail-Over move several FS between positions. Note: Be sure to properly identify FS for tracking purposes. | Part of the positions are still operating and the system can be configured to full operations |  |  |  |  |
|  | Turn off the power to a thin client and measure the time to replace it_____________ | The other workstations are still operating |  |  |  |  |
|  | Disconnect a workstation display from the LAN and measure the time to replace________________ | The other displays are still operating |  |  |  |  |
|  | Disconnect a keyboard and measure the time to replace______________ |  |  |  |  |  |
|  | Disconnect a mouse and measure the time to replace____________ |  |  |  |  |  |
|  | For the Port Server, create a fault by disconnecting the LAN cable and measure the time to isolate the fault and replace the Port Server Time to replace:__________ |  |  |  |  |  |
|  | Create a Display fault by disconnecting the power cable, measure the time to isolate remove and replace Time to fix the problem:_______ |  |  |  |  |  |
|  | Create a KVM Display fault by disconnecting the power cable, measure the time to isolate and replace the LRU:_______ |  |  |  |  |  |
|  | Calculate MTTR based on weighted average of each LRU in the system (See Data Reduction and analysis below) | MTTR<=30 minutes |  |  |  |  |
|  | Successful completion of M-Demo where all the LRU were fixed and operating properly will verify [AEFS295] | The technician is able to perform fault diagnosis on off-line components, replacement of off-line components, and verification of proper operation after repair, without impacting the AEFS system availability. |  |  |  |  |